Classified

Project: Titan "Bravo-Tango 7274"

From:     State Department, National Security

To:             Subject 49

Date:       Today

Hello Subject 49, if you are reading this it's because you have been choosen to take part on construction of the ultimate weapon: a war ready, single-seater giant exoskeleton.

Below i will guide you how to assembly of the titan and give you tips and instructions

Product Specifications:

Height:                                  <19 cm (normal standing)

Weight:                                              +/-  90 grams

Total Print Time:                 +/-  1 weeks

Difficulty:                             ★★★★☆

Crash test:                           ★★★☆☆

Nuclear Weapons:             No

Front, side and back of the titan

It is a highly articulate model almost entirely made by 3D printed components you will mount piece by piece.

I was able to print it with a cheap prusa i3 printer, so I'm sure you'll be able to print your Titan in about a week with normal PLA. You can also create the titan by using a 3D print internet services like shapeways.com or materialise.com or local 3D print services.

The Titan consists of a total of 52 pieces, which we divide into three categories: arm, frame, legs.

- Arm: For a single arm (green), which includes fingers, hand, forearm arm and various connections, serve 18 pieces, which are multiplied by 2 to make up the other arm.

- Frame: the frame (red) is composed of only 3 pieces, or the frame, "the eye" and a vertebra that serves to bend the titan to the right and left;

- Legs: From the pelvis down are included in the leg category (Yellow) the 9 final pieces.

What you need

You may need some tools to work the pieces, remember that most of the pieces are very delicate, and you may need to levigate them to join the pieces togheter.

Here a short list of what was useful to me for assembly: clamp, claw, glue gel, staples, rasps, sand paper, paper tape, spray cans for colouring, little Hammer, flat screw driver, little cutter.

The more is the time you work on it (especially for painting) the better is the final result.

TIPS TO IMPROVE STRENGTH OF 3D PRINT PARTS

- Print slowly (30/45 - mm/s);

- Use 1.5 layer height;

- Less power of cooling fan;

- The bigger is the nozzle diameter, the stronger are 3D print parts, but be aware of loosing details and accuracy, i suggest a standard 0.4 nozzle;

- Care the orientation of the print part from the slicer (you don't need to do it to print the titan, all the files are yet ready).

First Part: fingers of the right hand

You can print all the pieces in the order you want. Here i explain you in my order.

I started by printing the fingers, 11 pieces in total for the first print, performed very slowly at about 20-30 mm/s. Obviously it is up to you to imagine better settings and an ideal positioning in the printing plate.

The difficult part, however, is to assemble the fingers together, and then to the hand.

Let's try now to assemble the fingers, here is what you might need:

- Staples for the stapler

- Needle

- Lighter

- Little Hammer

- Small pincer

- Clamp

- Gummy glue gel, or vinyl glue (ATTENTION, this is a specific type of glue that becomes like rubber, that has the only purpose of slowing movements of moving parts)

- Fingers

Load the file DXHAND to print a complete hand, use low speed and low fan cooling, no support needed.

- To print the left hand, mirror the DXHAND file on X axis from your slicer (Cura and a lot of other slicer can easily do it) and print it.

Then start by checking if there are holes where to fit the staples, if the holes are obstructed by the printing material so take a needle, heat it for a few seconds using a lighter, pass it through the holes to unlock them.

Then if you have glue gel, use it to connect and slow the corresponding components to each other.

Take now a staple and insert it between the holes so as to connect the various parts of the finger between them, cut the excess with a pincer, and tighten around the piece with a clamp, taking care not to damage the pieces. Repeat until you have joined all the pieces.

- Hand Connectors

To connect the hand to the forearm, just join the pieces as shown in the following image:

The hand should then rotate inside the housing thanks to this fitting, and bend inside it.

Obviously, due to the fragility of the components, please pay attention and drive the movements of the hand and fingers from the source.

- Forearm and Arm

Next file is ARM, enable support. Print the same file another time again, to make the other arm.

Later connect the arm and the forearm to each other and insert the shorter Cylinder you will find on LITTLEPIECES file.

- Shoulder

In the Shoulder has been designed a cardan joint (gimbal) to move the arm in all directions as a real arm would do, through the rotation of three components among them that you can see in the image:

Then insert the components in this way, and if you want, apply the glue gel to slow down the movements. The fitting (the one that moves in red) is then pushed into the hole of the right arm of the frame.

They are a very fragile elements! So pay attention when you assemble them: use a rounded and pointed rasp to enlarge the holes of cardan joint, in order to let them join perfectly to the cardan ball and avoid fractures on parts.

After assembly the arm, test if all the movements are going correctly.

If you need to remove the shoulder joint from the frame, help yourself inserting a flat screwdriver between them and turn it.

Part Two: Frame, Eye and Vertebra

Now comes FRAME file.

The frame piece is about over 7-10 hours of printing. DON'T change the position you find yet and enable support on all the place.

Be very careful to position supports and check the slicer layer by layer as the print evolves. Especially in the base, in the central part and the first to be printed, the one that will later host the vertebra that will be slipped into it.

- Eye and Vertebra

Now print LITTLEPIECES file, which includes: eye, vertebra and cylinders.

Enable support, print slowly, use less speed cooling fan.

The eye can rotate up and down on his position, the vertebra connect the frame to hip bone and make the frame bend and freely rotate the frame right and left. Cylinder need to connect arm to forearm and leg parts to eachothers.

Eye: print it looking up when you finished printing the eye do not insert it immediately into the two supports of the frame, since it can't be removed easily, color it before inserting it if you want. Observe if you need the images to understand the right verse to insert it.

Third Part: Legs and all the rest

Printing the HIPBONE file.

An interesting part of the connectionof this file, as you can see from the image below, the connection of internal sphere of femurs to external sphere of hip bones in which it rotates inside, supported during the printing by a very small pillar which is then broken at the end of the print simply by moving the femurs

After putting the supports in the way you think is better there is only one important thing to do: rectify the infill offset so you can correctly print what will be the accommodation of the vertebra on the leg side. I set the infill for 5% lines, and set the offset y infill, on Cura, to 8, as you can see from the image:

The red circle shows the correct positioning of the infill offset where it will form the housing in which the vertebra is connected

When everything has been completed you can already connect the vertebra with the hip bone  through the hole in the upper part of the hip bone.

If you need to remove the vertebra from the hip bone, help yourself with a flat screwdriver and turn lever.

To improve stability use paper tape to slow movement of Hip bone, see the picture below.

- Leg 1 and 2, Foot and Foot Connector

Open the file LEG, enable support, print two times for the other leg.

So print the last parts of the titan is not very difficult and the image below shows to you where to fit the Cylinders where is shown by yellow arrows on the left. On the right side of the image you can see how to connect the foot to the Tibia by Foot Connector, which allows the foot to rotate in two axes.

in the image you can see where to place the Cylinders and where to slide to join the foot and its connector

Here the logo i attached on chest of my titans

Finished the Mounting, Next: Painting

If all goes well, you can already see your little titan standing up. Now you arrived here, you cannot not colour this huge work!

Just few tips for painting

Here i will tell how i did, of course you are free to make as you prefere to paint it, maybe you can try with oil colours, but i used spray cans.

Consider about the film of paint gives another layer of thickness on the titan, and some part may be harder to join together after painting.

To start:

- Use sand paper and rasps for levigation first! It is very recommended a good and deep levigation for a nice final result;

- Create a base color, i started with red;

- Mask all the piece to exspose the part that that have to be painted of another colour using paper tape. It is a long job, but, if you do it right, give a very good result.

If you are reading this means you got this thing seriously and i am grate and happy you did. I hope you had what you expected from this action figure. Thank you subject 49!

The last release update a lot of improvement. The difference compared to first prototype is huge!